ROBOTICS SOFTWARE

Firmware · ROS / IK Stack · HMI

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The software stack of the delta robot spans embedded firmware (servo loop, kinematics, communication), middleware (ROS2 or custom real-time framework), and HMI / supervisory software. Each layer must be documented, version-controlled, and linked to the hardware it runs on.

## 1.1 Metadata

| System | Delta Robot Software Stack |
| --- | --- |
| Category | Software |
| Subcategory | Embedded Firmware + Motion Control |
| Language | C (firmware), Python / C++ (ROS2 nodes) |
| RTOS | FreeRTOS v10.4 (STM32H7) |
| Middleware | ROS2 Humble / micro-ROS |
| Version Control | Git — repo: github.com/team/delta-robot |
| Document Type | Software Architecture / API Reference |
| Collected By | Team Member — Software Lead |
| File Name | Software_Custom_DeltaRobot_SWArchitecture.pdf |

# 2. Software Architecture

Layer Overview

| Layer | Component | Language | Runs On |
| --- | --- | --- | --- |
| L1 — HAL | STM32 HAL + custom BSP | C | STM32H743 bare-metal |
| L2 — RTOS | FreeRTOS tasks (servo, comms, safety) | C | STM32H743 |
| L3 — Kinematics | Delta IK/FK library | C / C++ | STM32 or host PC |
| L4 — Middleware | micro-ROS agent / ROS2 nodes | C++ / Python | Linux PC or STM32 |
| L5 — Application | Pick-and-place state machine | Python (ROS2) | Linux PC |
| L6 — HMI | Web dashboard (React + MQTT) | JS / Python | Linux PC / tablet |

# 3. Real-Time Servo Loop (L2)

## 3.1 Task Structure

| Task Name | Period | Priority | Function |
| --- | --- | --- | --- |
| ServoLoopTask | 500 µs | Highest (6) | PID close loop, PWM update |
| EncoderTask | 500 µs | High (5) | Read QEI counters, compute velocity |
| SafetyTask | 1 ms | High (5) | E-stop monitor, fault check |
| CommTask | 5 ms | Medium (3) | UART / CAN RX-TX |
| TelemetryTask | 50 ms | Low (1) | Log data to SD / USB CDC |

# 4. Delta Inverse Kinematics

## 4.1 Coordinate System

- Origin: centre of base platform, Z pointing downward.

- Joint angles θ1, θ2, θ3 measured from horizontal plane (positive = down).

- End-effector position: (x, y, z) in millimetres.

## 4.2 IK Function Signature

int delta_ik(float x, float y, float z, float *theta1, float *theta2, float *theta3);

Returns 0 on success, −1 if (x,y,z) is outside the reachable workspace. Input in mm, output angles in radians.

## 4.3 Forward Kinematics

int delta_fk(float t1, float t2, float t3, float *x, float *y, float *z);

# 5. ROS2 Node Graph

| Node | Topic / Service | Direction | Description |
| --- | --- | --- | --- |
| /delta_driver | /joint_states | Pub | Motor encoder positions |
| /delta_driver | /joint_commands | Sub | Target joint angles |
| /delta_kinematics | /cartesian_goal | Sub | Target (x,y,z) pose |
| /delta_kinematics | /joint_commands | Pub | Computed IK angles |
| /gripper_node | /gripper_cmd | Sub | Open / Close command |
| /hmi_node | /robot_status | Sub | Visualisation and logging |

# 6. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| SW Architecture Document | Required | Collect | Software_Custom_DeltaRobot_SWArchitecture.pdf |
| API Reference (IK library) | Required | Collect | Software_Custom_DeltaRobot_IK_API.pdf |
| FreeRTOS Task Table | Required | Collect | Software_Custom_DeltaRobot_RTOSTasks.pdf |
| ROS2 Node Graph | Preferred | Collect | Software_Custom_DeltaRobot_ROSGraph.pdf |
| Git Repository README | Preferred | Collect | Software_Custom_DeltaRobot_README.md |
| Unit Test Results | Preferred | Collect | Software_Custom_DeltaRobot_Tests.pdf |
| Deployment / Flashing Guide | Advanced | Collect | Software_Custom_DeltaRobot_Deploy.pdf |